# PRESS RELEASE

Würth Elektronik eiSos presents USB Type-C™ Development Kit for fast and easy migration to Type-C

Uses USB Type-C and Power Delivery controllers from STMicroelectronics

Waldenburg (Germany), 8 February 2019 – With the USB Type-C Development Kit from Würth Elektronik eiSos, those looking to take advantage of the vastly expanded range of options offered by USB Type-C and USB Power Delivery in their projects have a convenient and immediate solution at hand: built from proven USB-Type C technology from STMicroelectronics, Würth Elektronik’s USB Type-C Development Kit is ready for immediate use in prototypes and represents a reference design for safe and flexible USB PD applications up to 100W.

USB Type-C connector is today being used by many devices. Whatever the final application is, there are many reasons for adopting USB-C connectors. Since it is reversible, it can be plugged into a device any which way. Able to carry up to 100 W charging power (from 5V/0.1A up to 20V/5.0A), it merges and is backward compatible with the functionality of both legacy Type-A and -B ports in a single one and even can change roles on the fly.

With the USB Type-C Development Kit, Würth Elektronik eiSos has relieved its customers of two of the biggest challenges in developing USB PD applications:

1)	A robust solution with embedded protection against short-circuit, overvoltage, overcurrent and electromagnetic interference ensured by using USB Type C connectors, inductive components and capacitors from Würth Elektronik eiSos and certified USB Power Delivery controllers from ST with built-in protection.

2)	A complete and ready-to-use solution with evaluation boards and a demonstration board for sink and source applications from 0,5W up to 100W. These boards can be used without any extra software or firmware for faster development time.

The USB Type-C Development Kit includes three assembled boards and a high current rating USB Type-C cable.

The first board is the 40W Source Evaluation Board with the STUSB4710 autorun controller and on-board DC/DC converter from ST. The STUSB4710 integrates all the digital and high-voltage analog circuitry needed to autonomously support full USB PD negotiation. It supports up to 5 Power Data Objects (PDO) and power-sharing applications thanks to its I²C interface. Typical applications are power-source applications such as AC adapters, power supplies, power hubs, smart plugs and DC-DC applications.

The second board is the 100W Sink Evaluation Board with the STUSB4500 autorun controller from ST. It allows straightforward migration from STD-B, micro-B or custom power plugs to a Type-C connector. This solution is ideal for automatically prompting a charging process – even if the battery is empty. The STUSB4500 supports up to 3 PDOs. Typical applications are electronic devices charged through USB Type-C connector.

The third board is the 100W Source Demo Board with the STUSB1602 – an analog front-end that is a companion chip of the STM32 microcontroller – and on board DC/DC converter. The STUSB1602 provides up to 5 PDOs. Typical applications include power-source applications such as AC adapters, power supplies, power hubs, smart plugs and DC-DC applications.

Würth Elektronik eiSos is an ST Authorized Partner member of the STMicroelectronics Partner program. The ST Partner Program enhances the ecosystem around ST’s broad portfolio of products helping end customers to reduce development effort and accelerate time to market relying on ST trusted partner’s complementary products or services.

The USB Type-C Development Kit is now available at local sales representatives.

Available images

The following images can be downloaded from the Internet in printable quality: http://www.htcm.de/kk/wuerth

Image source: Würth Elektronik eiSos The USB Type-C Development Kit from Würth Elektronik eiSos and STMicroelectronics gives developers the decisive edge in USB-PD projects.

About Würth Elektronik eiSos

Würth Elektronik eiSos GmbH & Co. KG is a manufacturer of electronic and electromechanical components for the electronics industry. In 2017, the company employed 6 700 people. Würth Elektronik eiSos is one of the largest European manufacturers of passive components and is active in 50 countries. Production sites in Europe, Asia and North America supply a growing number of customers worldwide.

The product range includes EMC components, inductors, transformers, RF components, varistors, capacitors, resistors, quartz crystals, oscillators, power modules, Wireless Power Transfer, LEDs, sensors, connectors, power supply elements, switches, push-buttons, connection technology, fuse holders and solutions for wireless data transmission.

Through its technology partnership with the Audi Sport ABT Schaeffler Formula E Team and its support for the Formula E racing series, the company demonstrates its innovative strength in eMobility 
(www.we-speed-up-the-future.com).

Würth Elektronik eiSos is part of the Würth Group, the world market leader for assembly and fastening technology.

Würth Elektronik: more than you expect!

Further information at www.we-online.com

| Further information: Würth Elektronik eiSos GmbH & Co. KG Sarah Hurst Max-Eyth-Strasse 1 74638 Waldenburg Germany Phone: +49 7942 945-5186 E-mail: sarah.hurst@we-online.de www.we-online.de | Press contact: HighTech communications GmbH Brigitte Basilio Brunhamstrasse 21 81249 Munich Germany Phone: +49 89 500778-20 Telefax: +49 89 500778-77 E-mail: b.basilio@htcm.de www.htcm.de |
| --- | --- |