# PRESS RELEASE

- High Stability, Low Phase Noise OCXO

Waldenburg (Germany), 26 June 2019 – IQD has launched a new high stability and low phase noise Oven Controlled Crystal Oscillator (OCXO) which also offers an excellent short term stability. The IQOV-220 delivers an exceptional frequency stability performance down to ±0.5ppb (parts per billion) over the full industrial temperature range of -40 to 85 degrees C with a short term stability (Allan Deviation) of 0.5ppt (tau=1s). With a holdover spec of 6µS over 24 hours, a close in phase noise of typically -140dBC/Hz @10Hz and far out phase noise of typically -160dBc/Hz @100kHz, the IQOV-220 is ideal for High performance synthesizers, Network Clocks, Radar & Satellite Communications.

The IQOV-220 is available in 10MHz and is housed in a 36 x 27mm industry standard hermetically sealed through-hole metal package. This Sinewave OCXO is capable of driving up to 50Ω loads and operates at 12V, consuming a maximum of 5W during warm up and 1.2W maximum at steady state @25 degrees C.

The new OCXO has a frequency adjustment on Pin 2 which enables the frequency to be pulled by ±0.4ppm (parts per million) with a control voltage of 0 to 8V which will be sufficient to cover 10 years ageing.

This new series forms part of an extensive range of OCXO’s available from IQD, which include extremely low phase noise performance and ultra-small packages. Full details on the IQOV-220 are available at www.iqdfrequencyproducts.com.

Please note that an export license will be required by customers outside of the United Kingdom.

Available images

The following images can be downloaded from the Internet in printable quality: http://www.htcm.de/kk/wuerth

Image source: IQD IQOV-220, New High Stability, Low Phase Noise OCXO

About IQD

Backed by over 40 years’ experience in the manufacture of frequency products, IQD is a recognised market leader in the frequency control market and part of the Würth Elektronik eiSos group, one of the leading European manufacturers of passive components. With active customers in over 80 countries, IQD offers one of the most comprehensive frequency product ranges available, from low cost commercial grade product to that used in high reliability industrial and automotive applications including: Quartz Crystals, Clock Oscillators, AEC-Q200 Crystals & Oscillators, VCXOs, TCXOs, OCXOs, GPS Disciplined OCXOs, and Rubidium Oscillators.

Further info at www.iqdfrequencyproducts.com

About Würth Elektronik eiSos Group

Würth Elektronik eiSos Group is a manufacturer of electronic and electromechanical components for the electronics industry and a technology company that spearheads pioneering electronic solutions. The company currently employs 8,300 staff and generated sales of 848 million euros in 2018. Würth Elektronik eiSos is one of the largest European manufacturers of passive components and is active in 50 countries. Production sites in Europe, Asia and North America supply a growing number of customers worldwide.

The product range includes EMC components, inductors, transformers, RF components, varistors, capacitors, resistors, quartz crystals, oscillators, power modules, Wireless Power Transfer, LEDs, sensors, connectors, power supply elements, switches, push-buttons, connection technology, fuse holders and solutions for wireless data transmission.

The unrivaled service orientation of the company is characterized by the availability of all catalog components from stock without minimum order quantity, free samples and extensive support through technical sales staff and selection tools.

Through its technology partnership with the Audi Sport ABT Schaeffler Formula E Team and its support for the Formula Student racing series, the company demonstrates its innovative strength in eMobility 
(www.we-speed-up-the-future.com).

Würth Elektronik eiSos is part of the Würth Group, the world market leader for assembly and fastening technology.

Würth Elektronik: more than you expect!

Further information at www.we-online.com

| Further information: IQD Frequency Products Ltd Rebecca Long Station Road Crewkerne Somerset TA18 8AR United Kingdom Phone: +44 1460 270270 E-mail: rebecca.long@iqdfrequencyproducts.com www.we-online.com www.iqdfrequencyproducts.com | Press contact: HighTech communications GmbH Brigitte Basilio Brunhamstrasse 21 81249 Munich Germany Phone: +49 89 500778-20 Telefax: +49 89 500778-77 E-mail: b.basilio@htcm.de www.htcm.de |
| --- | --- |