# PRESS RELEASE

IQD Frequency Products has launched new OCVCSO

Ultra-Small Low Noise Low Power OCVCSO

Waldenburg (Germany), 21 February 2020 – IQD Frequency Products has launched a range of high frequency Oven Controlled Voltage Controlled SAW Oscillators (OCVCSOs). Three frequencies are currently available in the OCVCSO series, 400MHz, 800MHz and 1.2GHz and they are packaged in a 25.4 x 22 x 13.2mm industry standard SMD package.

These OCVCSOs have a noise floor of 10dB to 15dB lower than an OCXO at the same frequency, enabling a much improved phase noise performance for applications such as radar detection systems. The 400MHz OCVCSO frequency is generated from a fundamental high-Q SAW resonator, whereas the 800MHz and 1.2GHz are generated using high frequency fundamental SAW resonator with a low noise frequency doubler. Multiplying the frequency only once enables superior phase noise performance, down to -168dBc/Hz at the floor level and jitter between 10kHz to 100MHz of <10fs. The 400MHz OCVCSO being based on a fundamental frequency has a phase noise performance of -174dBc/Hz at the floor level and jitter of <5fs.

With stabilities down to ±2ppm over an operating temperature range of 0 to +50 degrees C and ageing of ±6ppm max over 10 years these new lightweight OCVCSOs, weighing only 8 grams, are ideal not only for radar but also simulators, communication systems and unmanned aerial vehicles (UAVs). The OCVCSO operates from a 5V supply with power consumption of between 0.8W to 1W, has a 50Ω sine output and a voltage control option, which enables the frequency to be pulled by a minimum of ±6ppm.

Further details are available on IQD’s website at www.iqdfrequencyproducts.com or for further advice, speak to a member of our Applications Support Team.

Available images

The following images can be downloaded from the Internet in printable quality: http://www.htcm.de/kk/wuerth

Image source: IQD IQD’s New Ultra Small Low Noise Low Power OCVCSO

About IQD

Backed by over 40 years’ experience in the manufacture of frequency products, IQD is a recognised market leader in the frequency control market and part of the Würth Elektronik eiSos group, one of the leading European manufacturers of passive components. With active customers in over 80 countries, IQD offers one of the most comprehensive frequency product ranges available, from low cost commercial grade product to that used in high reliability industrial and automotive applications including: Quartz Crystals, Clock Oscillators, AEC-Q200 Crystals & Oscillators, VCXOs, TCXOs, OCVCSOs & OCXOs, GPS Disciplined OCXOs, and Rubidium Oscillators.

Further info at www.iqdfrequencyproducts.com

About Würth Elektronik eiSos Group

Würth Elektronik eiSos Group is a manufacturer of electronic and electromechanical components for the electronics industry and a technology company that spearheads pioneering electronic solutions. The company currently employs 7,300 staff and generated sales of 822 million euros in 2019. Würth Elektronik eiSos is one of the largest European manufacturers of passive components and is active in 50 countries. Production sites in Europe, Asia and North America supply a growing number of customers worldwide.

The product range includes EMC components, inductors, transformers, RF components, varistors, capacitors, resistors, quartz crystals, oscillators, power modules, Wireless Power Transfer, LEDs, sensors, connectors, power supply elements, switches, push-buttons, connection technology, fuse holders and solutions for wireless data transmission.

The unrivaled service orientation of the company is characterized by the availability of all catalog components from stock without minimum order quantity, free samples and extensive support through technical sales staff and selection tools.

Through its technology partnership with the Audi Sport ABT Schaeffler Formula E Team and its support for the Formula Student racing series, the company demonstrates its innovative strength in eMobility 
(www.we-speed-up-the-future.com).

Würth Elektronik eiSos is part of the Würth Group, the world market leader for assembly and fastening technology.

Würth Elektronik: more than you expect!

Further information at www.we-online.com

| Further information: IQD Frequency Products Ltd Rebecca Long Station Road Crewkerne Somerset TA18 8AR United Kingdom Phone: +44 1460 270270 E-mail: rebecca.long@iqdfrequencyproducts.com www.we-online.com www.iqdfrequencyproducts.com | Press contact: HighTech communications GmbH Brigitte Basilio Brunhamstrasse 21 81249 Munich Germany Phone: +49 89 500778-20 Telefax: +49 89 500778-77 E-mail: b.basilio@htcm.de www.htcm.de |
| --- | --- |